MT/LT Series 500-2000VA Standby UPS

Please strictly observe all warnings and operating instructions in this manual. Keep this manual in a suitable place so that you can read the following notes carefully before installation. Before operating this product, be sure to read all safety notices and operating instructions carefully.

★ Foreword

Thank you for purchasing this series of UPS. It provides short-circuit protection, filtering and automatic shutdown, greatly enhancing the protection and safety of your equipment! Please read these instructions carefully to avoid mistakes during use.

★ Safety Precautions

To ensure safe use, please observe the following:

- This product is designed specifically for personal computers and is not recommended for life-support systems or other specific critical equipment.

- Do not exceed the rated load.

- The UPS contains a large-capacity battery. Never open the enclosure, as this may cause electric shock and other hazards. For internal servicing or battery replacement, take the unit to a designated service center.

- An internal short circuit in the UPS may cause electric shock or fire. Do not place containers holding liquid on top of the UPS, to avoid electric shock and other hazards.

- If the unit emits smoke, cut off the power immediately and contact your dealer.

- Do not store or use this product in the following environments:

◆ Places with flammable gases, corrosive substances or heavy dust

◆ Places with abnormally high or low temperature (above 40°C or below 0°C) or high humidity (above 90%)

◆ Places exposed to direct sunlight or near heating appliances

◆ Places subject to severe vibration

◆ Outdoors

- If a fire breaks out around the UPS, use a dry powder extinguisher. Using a liquid extinguisher may cause electric shock.

- Place the socket near the UPS so that in an emergency the plug can be pulled out to cut off the power

★ Installation and Use

1) Packaging Check

Open the package; it should contain:

- UPS unit

- User Manual

Take out the UPS and check whether the unit was damaged in transit. If it is damaged or parts are missing, do not switch it on; notify the carrier or your dealer.

2) UPS Placement

Place the UPS in a suitable location (see item 6 of Safety Precautions).

3) Mains Connection

When the input power cord connects the UPS to the mains, the battery starts charging. The best practice is to charge the battery for at least 6 hours before use.

4) Connecting Equipment

Connect the input cords of the equipment to the regulated battery output sockets on the rear panel of the UPS.

Reminder: Make sure the UPS is switched on, so that when the mains fails the UPS can protect the data of connected critical equipment from loss.

Caution: Do not connect printers or scanners to the regulated battery output sockets of the UPS; such devices will noticeably overload the UPS and prevent it from working properly.

5) Turning On the UPS

To switch on the UPS, press the power button lightly; the indicator lights up.

This unit features DC start-up: it can start the UPS directly without mains power, for convenient use.

Once the mains is interrupted and the UPS switches to backup supply, save your work on the computer and other loads or take other power-loss precautions promptly. After the mains recovers, turn the UPS on again in time so that the battery charges.

6) Turning Off the UPS

To switch off the UPS, press the power button again; the indicator goes out.

Caution:

- When switching on, turn on the UPS first and then the computer and other loads; when switching off, turn off the loads first and then the UPS.

- In general, it is recommended to leave the UPS switched on in order to keep the battery charged.

- If a problem occurs during operation, refer to the Troubleshooting section of this manual. If it cannot be resolved, contact professional customer service immediately.

Scan the QR code on the unit to obtain our contact details

*

Buzzer and Indicator Meanings

Indicator (single light): ● on  ○ off  ★ flashing

| Indicator | Buzzer Sound | Output | Description | Status |
| --- | --- | --- | --- | --- |
| ● | None | Normal | Mains input normal; UPS running normally | Normal |
| ★ | Every 10s | Normal | Mains input interrupted; UPS powered by battery | Normal |
| ★ | Every 1s | Normal | UPS battery supply reached low voltage | Normal |
| ○ | None | Stopped | Power switch off; UPS stops running | Shutdown |
| ● | Continuous | Stopped | UPS fault | Fault |

Indicator (three lights): ● on  ○ off  ★ flashing

| Indicator | Buzzer Sound | Output | Description | Status |
| --- | --- | --- | --- | --- |
| ● Green | None | Normal | Mains input normal; UPS running normally | Normal |
| ★ Amber | Every 10s | Normal | Mains input interrupted; UPS powered by battery | Normal |
| ★ Amber | Every 1s | Normal | UPS battery supply reached low voltage | Normal |
| ○ | None | Stopped | Power switch off; UPS stops running | Shutdown |
| ● Red | Continuous | Stopped | UPS fault | Fault |

★  Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| LED not lit. | The UPS is not switched on. | Press the UPS power switch again to confirm. |
|  | Battery voltage too low | Charge the battery for at least 6 hours |
|  | Battery damaged | Replace the battery |
| The UPS is always in battery mode | Input cord not connected properly | Re-insert the UPS input power cord firmly |
| The UPS keeps beeping | Load overload | Unplug some unnecessary loads. Before reconnecting equipment, make sure the model and capacity of the load match the load specification of the UPS. |
| Backup time too short | Battery voltage too low | Charge the battery for at least 6 hours |
|  | Load overload | Unplug some unnecessary loads. Before reconnecting equipment, make sure the model and capacity of the load match the load specification of the UPS. |
|  | Battery damaged | Replace the battery |

★ Specifications

220V Lead-Acid Battery Standby UPS

| Model | MT500A | MT650 | MT1000B | MT1000 | MT1500 | MT2000 |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity | 500VA/300W | 600VA/360W | 1000VA/600W |  | 1500VA/900W | 2000VA/1200W |
| Input Voltage Range | 180-270V | 140-300V AC |  |  |  |  |
| Output Voltage | 220V±10% (battery mode) |  |  |  |  |  |
| Transfer Time | Typically 4-8 ms |  |  |  |  |  |
| Output Waveform | Simulated sine wave |  |  |  |  |  |
| Battery & Quantity | 4.5AHx1 | 7AHx1 | 9AHx1 | 7AHx2 | 9AHx2 |  |
| Charging Time | Fully charged to 90% in 6 hours |  |  |  |  |  |
| Dimensions mm (D×W×H) | 280*100*144 |  |  | 370*138*165 |  |  |
| Weight (kgs) | 3.3 | 4.2 | 5.0 | 9.6 | 11.5 | 16.8 |

220V Lithium Battery Standby UPS

| Model | LT500 | LT1000(-PRO) | LT1500(-PRO) |
| --- | --- | --- | --- |
| Capacity | 600VA/360W | 1000VA/600W | 1500VA/900W |
| Input Voltage | 220VAC |  |  |
| Input Voltage Range | 140~300VAC |  |  |
| Output Voltage | 220V±10% (battery mode) |  |  |
| Transfer Time | Typically 4-8 ms |  |  |
| Output Waveform | Simulated sine wave |  |  |
| Battery Type | LiFePO4 lithium battery |  |  |
| Battery Capacity | 12V/15AH | 24V15Ah | 24V25AH |
| Charging Time | Fully charged to 90% in 8 hours |  |  |
| Dimensions mm (D×W×H) | 280*100*144 | 370*138*165 |  |
| Weight (kgs) | 3.4 | 7.4 | 9.5 |

110V Lead-Acid Battery Standby UPS

| Model | MT500-110V | MT1000V-110V(-PRO) |  | MT1500V-110V(-PRO) |
| --- | --- | --- | --- | --- |
| Capacity | 600VA/360W | 1000VA/600W |  | 1500VA/900W |
| Input Voltage | 110V AC |  |  |  |
| Input Voltage Range | 81~134V AC |  |  |  |
| Output Voltage | 110V±10% (battery mode) |  |  |  |
| Transfer Time | Typically 4-8 ms |  |  |  |
| Output Waveform | Simulated sine wave |  |  |  |
| Battery & Quantity | 12V/7Ah X 1 |  | 12V/7Ah x 2 | 12V/9Ah x 2 |
| Charging Time | Fully charged to 90% in 6 hours |  |  |  |
| Dimensions mm | 280*100*144 |  | 370*138*165 |  |
| Weight (kgs) | 4.2 |  | 10 | 12 |

110V Lithium Battery Standby UPS

| Model | LT500-110V | LT1000-110V(-PRO) | LT1500-110V(-PRO) |
| --- | --- | --- | --- |
| Capacity | 600VA/360W | 1000VA/600W | 1500VA/900W |
| Input Voltage | 110VAC |  |  |
| Input Voltage Range | 81~134VAC |  |  |
| Output Voltage | 110V±10% (battery mode) |  |  |
| Transfer Time | Typically 4-8 ms |  |  |
| Output Waveform | Simulated sine wave |  |  |
| Battery Type | LiFePO4 lithium battery |  |  |
| Battery & Quantity | 12V/15Ah | 24V15Ah | 24V25AH |
| Charging Time | Fully charged to 90% in 8 hours |  |  |
| Dimensions mm (D×W×H) | 280*100*144 | 370*138*165 |  |
| Weight (kgs) | 3.5 | 8 | 9.5 |

Environmental Parameters

| Temperature and Humidity | 0-90% humidity |
| --- | --- |
| Noise | Less than 40dB |

* Specifications are subject to change without prior notice.

* For detailed product information, scan the QR code on the unit

· Declaration: This is a Class A product. In a domestic environment this product may cause radio interference, in which case the user may be required to take adequate measures.

76.06.00007-00